J & J CUSTOM HOMES, LLC

133 West Washington Street, Suite D  |  Monticello, GA 31064  |  678-205-9383  |  www.jnjcustomhomes.com

SUBCONTRACTOR BID PACKAGES

Guarino Residence

| Project | Guarino Residence |
| --- | --- |
| Address | Lot 11 River Cove Meadows, Monticello, GA |
| County | Newton County |
| Project Type | Custom New Home — 2,637 SF Heated (single-story, slab-on-grade; +1,204 SF conditioned garage, 347 SF covered porch) |
| Project ID | PR-111 |
| Est. Duration | 10 months |
| Builder | J & J Custom Homes, LLC |
| Revision Date | June 25, 2026 |

This document contains 30 individual scope-of-work packages for the Guarino Residence. Each package defines the work, quantities, applicable codes, and quality requirements for one trade. Quantities are derived from the J & J estimate and takeoff; all subcontractors must verify field dimensions before ordering or installing.

PACKAGES INCLUDED

- 01 — Site Work & Earthwork

- 01B — Silt Fence & Erosion Control

- 02 — Septic System

- 04 — Propane & Gas Supply

- 05 — Foundation & Monolithic Slab

- 07A — Framing Lumber — Materials

- 07B — Framing Labor

- 07C — Engineered Floor System — Materials

- 08 — Roofing & Gutters

- 09 — Plumbing

- 10 — HVAC & Interior Gas

- 11 — Electrical

- 12 — Insulation

- 13 — Drywall

- 14 — Exterior Finishes — Metal Cladding

- 15 — Windows & Exterior Doors

- 16 — Interior Trim & Carpentry

- 17 — Painting & Exposed-Structure Coating

- 18 — Tile

- 19 — Cabinetry

- 20 — Countertops

- 21 — Flooring — Stained & Sealed Concrete

- 22 — Masonry — Stone Veneer

- 23 — Glass & Shower Enclosures

- 24 — Garage Doors

- 26B — Driveway & Concrete Walkways

- 27 — Landscaping

- 28A — Final Clean — Interior

- 28B — Window Cleaning

- 28C — Exterior Pressure Washing

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 01 — Site Work & Earthwork | Clearing · Grading · Construction Drive |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish all labor, equipment, and materials to clear, strip, and rough-grade the building site and construct a stable gravel construction drive. Lot is a pasture (subdivision corner lot) — no tree clearing or demolition anticipated. Silt fence and erosion controls are a separate package (Scope 01B).

SCOPE INCLUDES

- Grubbing and mulching of the building pad and drive corridor.

- Lot clearing and excavation / rough cut-and-fill to establish building pad.

- Construct concrete wash-out pit per local code.

- Set culvert pipe at drive entrance and install driveway mat.

- Place #34 gravel construction drive to house site, 12 ft wide (≈1,320 SF).

SCOPE EXCLUDES

- Silt fence and temporary erosion/sediment controls (Scope 01B).

- Final grading, backfill, sod, and landscaping (Scope 27).

- Final concrete driveway and walkways (Scope 26B).

- Footing/foundation excavation and slab gravel (Scope 05).

- Septic system and drain field (Scope 02); utility trenching for water/electric.

- Boundary survey and erosion-control plan engineering (by others).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Grubbing and mulching | 1 | Days | JJCH cannot be sure on exact amount of mulching, so this is an allowance. Anything above and beyond estimated will be billed at cost plus 20% via a change order. Invoice/receipt will be provided at your request — 1 Days |
| Lot Clearing & Excavation | 2 | Days | JJCH cannot be sure on exact amount for tree removal, so this is an allowance. Anything above and beyond estimated will be billed at cost plus 20% via a change order. Invoice/receipt will be provided at your request — 2 Days |
| Concrete wash out pit | 1 | each | Provide a safe area for concrete trucks to wash out in accordance with local building code — 1 each |
| Culvert Pipe | 1 | each | Culvert Pipe — 1 each |
| Driveway mat | 110 | LF | Driveway mat — 110 LF |
| #34 Gravel to house site - 12' wide | 1320 | sqft | JJCH cannot be sure on exact amount of gravel, so this is an allowance. Anything above and beyond estimated will be billed at cost plus 20% via a change order. Invoice/receipt will be provided at your request — 1,320 ft2 (incl. 10% waste) |

SPECIAL NOTES

NOTE: Grubbing/mulching and lot clearing are carried as allowances — quantities above estimate are billed at cost plus 20% via signed change order. Confirm disturbed-area limits before mobilizing. Erosion controls (Scope 01B) must be in place before ground disturbance.

APPLICABLE CODES & STANDARDS

- 2018 IRC Chapter 18 — Soils and Foundations

- Georgia Erosion and Sediment Control Act (OCGA 12-7)

- NPDES Construction General Permit — Georgia EPD

- Newton County Land Disturbance Permit requirements

- OSHA 29 CFR 1926 Subpart P — Excavation Safety

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm property lines, building corners, and finished-floor elevation with J & J.

- Confirm land-disturbance permit is issued before any ground disturbance.

- Locate and mark all existing utilities (call 811) before digging.

COMPLETION — Before calling for inspection or payment

- Building pad cut to correct elevation and compacted; drains positively away from pad.

- Construction drive passable for concrete and delivery trucks in wet weather.

- Disturbed area ready; erosion controls (Scope 01B) confirmed in place.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 01B — Silt Fence & Erosion Control | BMPs · Silt Fence · NPDES |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish all labor and materials to install and maintain temporary erosion and sediment control best-management-practices (BMPs) — silt fence on all downhill slopes of disturbed areas — in accordance with the approved erosion-control plan and the NPDES Construction General Permit. Controls must be in place before any ground disturbance.

SCOPE INCLUDES

- Install silt fence on all downhill slopes of disturbed areas, per local code (≈750 LF).

- Trench in and anchor silt fence; install supplementary BMPs as the plan requires (inlet protection, check dams, construction exit).

- Inspect and maintain BMPs through construction; repair after storm events.

- Remove temporary controls at final stabilization.

SCOPE EXCLUDES

- Clearing, grading, and construction drive (Scope 01).

- Concrete wash-out pit (Scope 01).

- Final grading, sod, and permanent stabilization (Scope 27).

- Erosion-control plan engineering and NOI/NOT filing (by others).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Temporary Erosion Control | 750 | LF | Install silt fence on all downhill slopes of areas cleared, in accordance with local building codes. — 750 ft |

SPECIAL NOTES

NOTE: Silt fence and BMPs must be installed BEFORE any land disturbance and maintained per the NPDES permit. Quantity (≈750 LF) is based on estimated disturbed-area perimeter — confirm actual disturbed limits before mobilizing.

APPLICABLE CODES & STANDARDS

- Georgia Erosion and Sediment Control Act (OCGA 12-7)

- NPDES Construction General Permit — Georgia EPD

- Georgia Manual for Erosion and Sediment Control (Green Book)

- Newton County — Land Disturbance Permit / erosion-control inspection

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm approved erosion-control plan and disturbed-area limits with J & J.

- Confirm land-disturbance permit is issued before installing controls.

- Confirm BMP locations (silt fence, inlet protection, construction exit).

COMPLETION — Before calling for inspection or payment

- Silt fence installed continuously on all downhill perimeters and trenched in.

- All required BMPs in place before ground disturbance begins.

- Passes initial county/EPD erosion-control inspection.

- Maintenance plan understood; repairs made after rain events.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 02 — Septic System | Design · Permit · Install · 4-Bath |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Licensed Georgia septic contractor to design, permit, and install a complete on-site sewage management system sized for a 4-bathroom residence, per Newton County Environmental Health specifications. Likely engineered/Level-3 system on a new subdivision lot.

SCOPE INCLUDES

- Soil/percolation evaluation and septic system design.

- Septic permit application and fees through Newton County Environmental Health.

- Supply and install tank, distribution box, and drain field/absorption lines.

- Excavation, bedding, backfill, and final grade over the field.

- Coordinate sewer stub-out location with plumber (Scope 09).

SCOPE EXCLUDES

- Interior plumbing and building sewer to the 5-ft line (Scope 09).

- Final landscape grading and seeding over the field (Scope 27).

- Public water service (Scope 09 — water meter/tap).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Sewer: Septic System, Plan, & Permit | 1 | each | Septic system installed by a GA licensed septic contractor, per Newton County specs. Allowance. — 1 each |

SPECIAL NOTES

NOTE: Carried as a $13,000 allowance. System type (conventional vs. engineered/pump) must be confirmed against the county-approved design; cost reconciled by change order once the permit is issued.

APPLICABLE CODES & STANDARDS

- Georgia DPH Rule 511-3-1 — On-Site Sewage Management Systems

- Newton County Environmental Health Department requirements

- 2018 IRC Chapter 32 — Plumbing reference

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm approved septic design and permit are in hand before excavation.

- Confirm field location does not conflict with drive, well/water line, or structure setbacks.

COMPLETION — Before calling for inspection or payment

- System installed per approved design; as-built provided to J & J.

- Passes Newton County Environmental Health final inspection.

- Tank locations and clean-outs documented for the owner.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 04 — Propane & Gas Supply | 1,000-Gal Buried Tank · Set & Bury |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish and set a 1,000-gallon buried propane tank, including excavation, anchoring, and backfill, and coordinate first fill. Gas piping to appliances is by the mechanical trade (Scope 10).

SCOPE INCLUDES

- Furnish 1,000-gallon propane tank (buried) — allowance.

- Excavate, set, anchor, and bury tank per NFPA 58 separation distances.

- Backfill and rough-grade over tank.

- Coordinate tank location and stub with the gas/mechanical trade and J & J.

SCOPE EXCLUDES

- Gas distribution piping from tank to appliances (Scope 10 — HVAC & Interior Gas).

- Final grade and landscaping over tank (Scope 27).

- Appliance and fireplace connections.

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Gas: Propane Tank (1,000 Gallon) | 1 | each | 1,000-gallon buried propane tank - allowance. — 1 each |
| Bury Propane tank | 1 | each | Bury Propane tank — 1 each |

SPECIAL NOTES

NOTE: Tank is carried as a $7,500 allowance plus $1,000 to bury. Confirm whether owner prefers gas tankless water heaters off this tank (currently estimated as electric heat-pump units).

APPLICABLE CODES & STANDARDS

- NFPA 58 — Liquefied Petroleum Gas Code

- NFPA 54 / 2018 IFGC — National Fuel Gas Code

- Georgia Public Service Commission — LP gas regulations

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm tank location meets NFPA 58 clearances from structure, drive, and ignition sources.

- Confirm utility/electric and septic locations to avoid conflict.

COMPLETION — Before calling for inspection or payment

- Tank set level, anchored, and buried to manufacturer spec.

- System leak-tested; first fill scheduled.

- Location documented for the mechanical trade tie-in.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 05 — Foundation & Monolithic Slab | Footers · 4" Slab · Vapor Barrier · Termite |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish all labor, equipment, and materials to form and pour the footings and a monolithic slab-on-grade foundation, including sub-slab gravel, vapor barrier, reinforcing, and termite pre-treatment. 3,000 PSI fiber-reinforced concrete.

SCOPE INCLUDES

- Form and pour footers (≈490 LF form board; ≈23 CY concrete; #4 rebar continuous).

- Place #57 sub-slab stone (≈6 truck loads) and 6-mil vapor barrier.

- Set mesh chairs and #4 rebar grid at 24" OC; pour 4" monolithic slab (≈58 CY).

- Labor to place, finish, and cure monolithic slab (≈3,841 SF).

- Pump truck for slab pour.

- Termite soil pre-treatment by a licensed applicator (≈$900).

SCOPE EXCLUDES

- Footing/pad excavation and rough grade (Scope 01).

- Under-slab plumbing and electrical rough-in (Scopes 09 / 11) — coordinate before pour.

- Slab staining/sealing as the finished floor (Scope 21).

- Exterior flatwork — drive, walks, porch slabs poured later (Scope 26B).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Form Boards - Footer | 490 | LF | Form Boards - Footer — 490 LF (incl. 10% waste) |
| Concrete - Footer | 23 | CY | Concrete - Footer — 23 yd (incl. 10% waste) |
| #4 Rebar 20" Sticks - Footer | 74 | LF | #4 Rebar 20" Sticks - Footer — 74 LF (incl. 10% waste) |
| Mesh Chairs 100 pack - Slab | 8 | sqft | Mesh chairs that keep the WWM in the center of the slab — 8 each |
| Concrete- 4 inches - Slab | 58 | CY | 3000 PSI fiber reinforced concrete — 58 YD3 (incl. 10% waste) |
| Labor - Monolithic slab | 3841 | sqft | Labor - Monolithic slab — 3,841 ft2 (incl. 10% waste) |
| Pump Truck - Slab pour | 2 | sqft | Pump truck required to pour the slab — 2 each |
| Vapor Barrier - Slab | 6 | Rolls | 6 Mill plastic vapor barrier — 6 Rolls (incl. 10% waste) |
| 1/2" Gravel - Slab | 6 | Truck load | #57 Stone under slab — 6 Truck load (incl. 10% waste) |
| Termite Protection | 1 | each | Termite Protection — 1 each |
| Slab reinforcing - #4 rebar grid 24in OC | 192 | sqft | Slab reinforcing steel. — 192 each |

SPECIAL NOTES

NOTE: The slab is the finished floor (stained/sealed — Scope 21). Finish flatness and freedom from cracking, stains, and control-joint defects are critical. Coordinate all under-slab rough-ins and inspect before pour. Slab reinforcing was upgraded to a #4 rebar grid.

APPLICABLE CODES & STANDARDS

- 2018 IRC Chapter 4 — Foundations (R401–R408)

- 2018 IRC R506 — Concrete slab-on-ground floors

- ACI 318 — Building Code Requirements for Structural Concrete

- Newton County — Foundation/slab inspection requirements

- OSHA 29 CFR 1926 Subpart Q — Concrete and Masonry Construction

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm building pad elevation, dimensions, and squareness with J & J.

- Verify all under-slab plumbing/electrical is installed, inspected, and protected.

- Confirm termite pre-treat is applied and documented before pour.

- Confirm vapor barrier is continuous and lapped before steel placement.

COMPLETION — Before calling for inspection or payment

- Footing and slab pass county inspection prior to and after pour as required.

- Slab finished flat and clean — suitable to receive stain/seal as the final floor.

- Control/saw joints cut per plan; no spalling or surface defects.

- Termite treatment certificate provided to J & J.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 07A — Framing Lumber — Materials | Supply Scope · J&J Will Purchase |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Supplier to furnish and deliver the framing lumber package for the residence. This is a material-supply scope for pricing only — J & J purchases the package. Framing labor is a separate scope (07B); the engineered floor system is a separate scope (07C).

SCOPE INCLUDES

- Framing lumber package — 2x4 walls, 2x8 ceiling joists, 2x6 rafters (SYP/SPF), ≈4,188 SF.

- All plates, headers, blocking, and miscellaneous dimensional lumber per plan.

- Connectors/hardware (Simpson Strong-Tie) per the connector schedule if included.

- Delivery to site, staged per J & J direction.

SCOPE EXCLUDES

- Engineered floor/roof system — I-joists/trusses (Scope 07C).

- Framing labor (Scope 07B).

- Sheathing/cladding (Scope 14); roofing (Scope 08).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Framing Lumber | 4188 | sqft | Standard 2x4 walls, 2x8 Ceiling Joist, 2x6 Roof rafter. Southern pine or spruce/fir — 4,188 ft2 |

SPECIAL NOTES

NOTE: Lumber pricing is volatile and date-stamped 2026-06-25 — confirm current pricing and lead time at award. Provide a takeoff-based package quote against the framing plans.

APPLICABLE CODES & STANDARDS

- 2018 IRC Chapters 5, 6, 8 — Floors, Walls, Roof-Ceiling Construction

- AWC Wood Frame Construction Manual (WFCM)

- PS 20 — American Softwood Lumber Standard (grade stamps required)

DELIVERY & RECEIPT CHECKLIST

PRE-DELIVERY — Confirm before shipping

- Confirm species/grade, package scope, and current pricing at award.

- Confirm connector/hardware schedule with the EOR.

- Confirm delivery date and staging with J & J.

RECEIPT — Required at delivery

- Package delivered complete and grade-stamped; quantities verified against ticket.

- Materials staged and protected from weather on site.

- Any shortages/back-orders reported to J & J immediately.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 07B — Framing Labor | Labor Only · J&J Supplies All Materials |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish all labor and equipment to frame the residence complete — walls, low-slope engineered roof/ceiling structure, beams, and blocking. J & J furnishes all framing lumber (Scope 07A) and the engineered floor system (Scope 07C); this scope is labor only.

SCOPE INCLUDES

- Frame all walls, set ceiling/roof structure, beams, headers, and blocking, ≈4,188 SF.

- Install all Simpson Strong-Tie connectors and hardware per the connector schedule.

- Set engineered floor/roof members furnished under Scope 07C.

- Install windows/exterior doors if directed (coordinate with Scope 15).

- Report any material shortages or defects to J & J immediately.

SCOPE EXCLUDES

- Framing lumber and engineered members — furnished by J & J (Scopes 07A / 07C).

- Metal wall cladding, fascia, and porch ceilings (Scope 14).

- Roofing membrane/panels (Scope 08).

- Interior trim, doors, and stairs (Scope 16); insulation (Scope 12).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Framing Labor - SF Price | 4188 | sqft | Framing Labor - SF Price — 4,188 ft2 |

SPECIAL NOTES

NOTE: Roof is low-slope (½:12 / 1:12) framed as an engineered open-span system. Exposed structure must be clean and consistent because ceilings are LEFT EXPOSED and sprayed black (Scope 17) — framing quality is a finished-appearance item, not just structural.

APPLICABLE CODES & STANDARDS

- 2018 IRC Chapters 5, 6, 8 — Floors, Walls, Roof-Ceiling Construction

- AWC Wood Frame Construction Manual (WFCM)

- Simpson Strong-Tie Connector Installation Guide

- Newton County — Framing inspection requirements

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm slab dimensions and squareness before plating.

- Confirm engineered roof/floor layout and connector schedule with the EOR.

- Confirm lumber and engineered package (Scopes 07A/07C) are on site and complete.

COMPLETION — Before calling for inspection or payment

- Structure framed plumb, level, and square; all connectors installed per plan.

- Exposed framing is clean and consistent (it remains visible in finished home).

- Passes county framing inspection.

- Ready to receive cladding, roofing, and rough-in trades.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 07C — Engineered Floor System — Materials | Supply Scope · J&J Will Purchase · I-Joists/Trusses |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Supplier to design, furnish, and deliver the engineered floor/roof system (wood I-joists or trusses) for the low-slope open-span structure, ≈4,850 SF. This is a material-supply scope for pricing only — J & J purchases the package; framing labor sets it (Scope 07B).

SCOPE INCLUDES

- Engineered wood I-joists or trusses for the low-slope roof/floor system, ≈4,850 SF.

- Sealed engineering layout, member design, and hangers/connectors as required.

- Rim board, bridging/strongbacks, and accessories per the engineered layout.

- Delivery to site coordinated with the framing schedule.

SCOPE EXCLUDES

- Dimensional framing lumber (Scope 07A).

- Framing labor / installation (Scope 07B).

- Roofing (Scope 08); insulation (Scope 12).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Engineered Floor | 4850 | sqft | Wood I-Joist or Trusses — 4,850 ft2 |

SPECIAL NOTES

NOTE: Priced as an open-span engineered system because rafters are dropped (no double-count). Provide a sealed layout and current pricing/lead time at award. Confirm whether I-joists or trusses best suit the low-slope exposed design.

APPLICABLE CODES & STANDARDS

- 2018 IRC R502 / R802 — Engineered wood floor & roof framing

- ANSI/TPI 1 — National Design Standard for Metal Plate Connected Wood Truss Construction

- APA / manufacturer (I-joist) engineering and installation specifications

DELIVERY & RECEIPT CHECKLIST

PRE-DELIVERY — Confirm before shipping

- Confirm structural layout, spans, and loads with the EOR.

- Confirm I-joist vs. truss selection for the exposed low-slope design.

- Confirm sealed layout, pricing, and lead time at award.

RECEIPT — Required at delivery

- Sealed engineered layout and members delivered complete; verified against ticket.

- Hangers/accessories included; staged and protected on site.

- Any shortages/back-orders reported to J & J immediately.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 08 — Roofing & Gutters | Standing-Seam Metal · Low-Slope · 6" K-Style |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish and install a standing-seam metal roof system, mechanically-seamed for low slope, turnkey, plus gutters and downspouts. Roof is nearly flat (½:12 / 1:12) and requires a low-slope-rated panel and underlayment assembly.

SCOPE INCLUDES

- Standing-seam metal roof, mechanically-seamed low-slope panels, ≈5,578 SF, turnkey (allowance).

- High-temp underlayment and all flashings, closures, ridge/eave trim.

- 6" K-style gutters (≈200 LF) and 3" downspouts (≈100 LF).

- All penetrations flashed watertight; coordinate with plumbing/HVAC vents.

SCOPE EXCLUDES

- Roof framing and decking (Scope 07).

- Fascia and metal wall cladding (Scope 14).

- Interior gutters/scuppers if drainage design changes (confirm).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Metal Roofing | 5578 | sqft | Standing-seam metal roof, low-slope, mechanically-seamed, turnkey installed. Allowance. |
| Gutters (6" Standard K-Style) | 200 | LF | Gutters (6" Standard K-Style) — 200 feet |
| Downspouts (3" Standard) | 100 | LF | Downspouts (3" Standard) — 100 feet |

SPECIAL NOTES

NOTE: Low-slope roof is the single largest line and biggest risk — confirm panel profile, seam type, and warranty, and obtain a low-slope-rated roofer quote ($9–13/SF range). Confirm whether parapet/internal drains or scuppers are required instead of K-style gutters.

APPLICABLE CODES & STANDARDS

- 2018 IRC Chapter 9 — Roof Assemblies (R901–R908)

- 2018 IRC R905.10 — Metal roof panels (low-slope assembly per mfr listing)

- 2018 IRC R903.3 — Gutters and downspouts

- NRCA Roofing Manual — Low-slope metal systems

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm low-slope panel system is listed/warranted for the actual roof pitch.

- Confirm decking/underlayment assembly with manufacturer.

- Coordinate all roof penetration locations before installation.

COMPLETION — Before calling for inspection or payment

- Roof watertight; water-test or inspect all seams, flashings, and penetrations.

- Gutters sloped to downspouts; downspouts discharge away from foundation.

- Manufacturer warranty documentation provided to J & J.

- Passes county roofing/dry-in inspection.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 09 — Plumbing | Water Tap · Rough-In · Fixtures · 4 Baths |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish all labor and materials for complete plumbing: public water tap/meter connection, under-slab and rough-in DWV and supply, water heaters, and set all fixtures for a 4-bathroom + kitchen + laundry residence. Fixture and fixture-trim items are owner allowances.

SCOPE INCLUDES

- Connect to public water main; meter and tap fee (allowance, billed at cost).

- Under-slab and above-slab rough-in: 20 fixture openings + 12 water openings (fridge, ice maker, washer, hose bibs).

- Two high-efficiency water heaters (allowance).

- Set all fixtures: kitchen sink + faucet + pot filler; 4 toilets; vanity sinks/faucets/drains (baths 1–4); master freestanding tub + valve; shower valves/heads (all baths).

- Hose bibs, washer box, and all trim/connections.

SCOPE EXCLUDES

- Septic system and building sewer beyond the 5-ft line (Scope 02).

- Gas piping to appliances/water heaters (Scope 10).

- Tile shower pans/mud beds (Scope 18); shower glass (Scope 23).

- Vanity cabinets and countertops (Scopes 19 / 20).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Water Meter | 1 | each | Connection to public water main, meter and tap fee. Allowance - billed at cost. — 1 each |
| Count Each Fixture Opening | 20 | each | Count Each Fixture Opening — 20 each |
| Count Each Water Opening | 12 | each | Fridge, Ice Maker, Washer, Hose Bibbs, etc. — 12 each |
| Water Heater | 2 | each | Two high-efficiency water heaters. Allowance. |
| Potfiller - Kitchen | 1 | each | Potfiller - Kitchen Allowance. |
| Kitchen Sink - Undermount | 1 | each | Kitchen Sink - Undermount Allowance. |
| Kitchen Sink - Faucet | 1 | each | Kitchen Sink - Faucet Allowance. |
| Toilet - Bath 1 | 1 | each | Toilet - Bath 1 Allowance. |
| Vanity Sink - Bath 1 | 2 | each | Vanity Sink - Bath 1 Allowance. |
| Vanity Sink - Faucet - Bath 1 | 2 | each | Vanity Sink - Faucet - Bath 1 Allowance. |
| Vanity Sink drain kit - Bath 1 | 2 | each | Vanity Sink drain kit - Bath 1 — 2 each |
| Shower valve and head - Allowance - Bath 1 | 2 | each | Allowance. |
| Freestanding tub - Bath 1 | 1 | each | Freestanding soaking tub, master. Allowance. |
| Tub Valve- Allowance - Bath 1 | 1 | each | Allowance. |
| Toilet - Bath 2 | 1 | each | Toilet - Bath 2 Allowance. |
| Vanity Sink - Bath 2 | 1 | each | Comes with vanity Allowance. |
| Vanity Sink - Faucet - Bath 2 | 1 | each | Vanity Sink - Faucet - Bath 2 Allowance. |
| Vanity Sink drain kit - Bath 2 | 1 | each | Vanity Sink drain kit - Bath 2 — 1 each |
| Shower valve and head - Allowance - Bath 2 | 1 | each | Allowance. |
| Toilet - Bath 3 | 1 | each | Toilet - Bath 3 Allowance. |
| Vanity Sink - Bath 3 | 1 | each | Vanity Sink - Bath 3 Allowance. |
| Vanity Sink - Faucet - Bath 3 | 1 | each | Vanity Sink - Faucet - Bath 3 Allowance. |
| Vanity Sink drain kit - Bath 3 | 1 | each | Vanity Sink drain kit - Bath 3 — 1 each |
| Shower valve and head - Allowance - Bath 3 | 1 | each | Allowance. |
| Toilet - Bath 4 | 1 | each | Toilet - Bath 4 Allowance. |
| Vanity Sink - Bath 4 | 1 | each | Vanity Sink - Bath 4 Allowance. |
| Vanity Sink - Faucet - Bath 4 | 1 | each | Vanity Sink - Faucet - Bath 4 Allowance. |
| Vanity Sink drain kit - Bath 4 | 1 | each | Vanity Sink drain kit - Bath 4 — 1 each |
| Shower valve and head - Allowance - Bath 4 | 1 | each | Allowance. |

SPECIAL NOTES

NOTE: Confirm electric heat-pump vs. gas tankless water heaters (propane is available on site — Scope 04). All fixtures are allowances; final selections and trim finishes to be confirmed before rough-in of valves.

APPLICABLE CODES & STANDARDS

- 2018 IPC — International Plumbing Code

- Georgia State Minimum Standard Plumbing Code

- 2018 IRC Chapters 25–32 — Plumbing

- Newton County — Plumbing permit and inspection

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm fixture selections, locations, and rough-in heights with J & J/owner.

- Confirm water heater type (electric vs. gas tankless) before rough-in.

- Coordinate under-slab rough-in and inspection BEFORE slab pour (Scope 05).

COMPLETION — Before calling for inspection or payment

- Rough-in passes inspection; pressure/DWV tests pass.

- All fixtures set, sealed, and operational; no leaks.

- Water heaters operational; hose bibs and washer box functional.

- Final plumbing inspection passed.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 10 — HVAC & Interior Gas | 2 Systems · Master Zone · Exposed Duct · Gas Lines |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish all labor and materials for complete HVAC: two systems (house + conditioned garage), a master-suite zone damper, all ductwork, and interior gas piping to appliances. Per the exposed-structure design, main ductwork is hard spiral/exposed and must be installed to a finished-appearance standard.

SCOPE INCLUDES

- House air handler and condensing unit, high-efficiency (allowance).

- Separate HVAC system for the conditioned garage.

- Hard spiral / exposed ductwork for main living areas (design intent: exposed).

- Flex duct work to garage; supply/return registers and grilles.

- Zoned damper to separate master-suite cooling.

- Interior gas piping to fireplace, furnace, tankless WH, ranges (2 connections).

SCOPE EXCLUDES

- Propane tank set and bury (Scope 04).

- Spray-coating exposed duct black (Scope 17 — painting).

- Electrical connections/disconnects to equipment (Scope 11).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Gas Lines | 2 | each | Fireplace, Furnace, Tankless WH, etc — 2 each |
| HVAC - Air Handler and Condensing Units | 1 | each | HVAC system for residence; high-efficiency. Allowance. |
| Flex Duct work | 1 | sqft | Hard spiral ductwork (exposed-structure design). — 1 ft2 |
| HVAC - Garage | 1 | sqft | HVAC for conditioned garage. — 1 sq ft |
| Garage - Flex Duct work | 1204 | sqft | Garage - Flex Duct work — 1,204 ft2 |
| HVAC zone damper - master suite | 1 | each | Zoned damper to separate master-suite cooling. — 1 each |

SPECIAL NOTES

NOTE: Main ductwork is EXPOSED and sprayed black (Scope 17) — runs must be straight, level, and laid out cleanly as a finished element, not hidden. Two systems are driven by the conditioned garage and master comfort. Get equipment + exposed-spiral-duct quote; this is a high-scatter line.

APPLICABLE CODES & STANDARDS

- 2018 IMC — International Mechanical Code

- 2018 IFGC — International Fuel Gas Code; NFPA 54

- Georgia State Minimum Standard Mechanical Code

- ASHRAE 62.2 — Ventilation and Indoor Air Quality

- Newton County — Mechanical and gas permit

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm exposed-duct routing and layout with J & J before fabrication (appearance matters).

- Confirm equipment selections, zoning, and load calc (Manual J/D).

- Coordinate gas appliance locations and propane tank stub (Scope 04).

COMPLETION — Before calling for inspection or payment

- Both systems operational; refrigerant charged; condensate drained properly.

- Exposed ductwork straight, level, and clean — ready for black spray-coat.

- Gas piping leak-tested and inspected.

- Master zone damper functional; final mechanical/gas inspections passed.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 11 — Electrical | 400A Service · Wiring · Lighting · Low-Voltage |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish all labor and materials for complete electrical: 400-amp service and panel, code-minimum wiring throughout, lighting, and low-voltage/security rough-in. Additional electrical beyond code minimum is billed at cost plus 20%.

SCOPE INCLUDES

- Code-minimum wiring on SF basis (≈3,841 SF); 200A service/panel base, upgraded to 400A.

- Permanent power service connection from transformer.

- Low-voltage, data, alarm, and camera rough-in (allowance).

- 45 can lights + wiring; 8 fans/lights; under-cabinet LED at kitchen.

- Vanity light fixtures at all 4 baths.

- Smoke/CO alarms, devices, plates, and panel labeling.

SCOPE EXCLUDES

- Light fixtures beyond those listed (decorative fixtures may be owner-supplied — confirm).

- HVAC equipment wiring beyond disconnects (coordinate with Scope 10).

- Exposed conduit feature wiring unless directed (industrial aesthetic — confirm).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Electrical - SF Price | 3841 | sqft | Code minimum electrical wiring. 200 Amp service and panel included. Additional electrical will be billed at cost plus 20% — 3,841 sq ft |
| Upgrade to 400 AMP service | 1 | sqft | Upgrade to 400 AMP service — 1 ft2 |
| Low Voltage, Alarm, Cameras - Allowance | 1 | each | Low-voltage, data, security rough-in - allowance. |
| Under cabinet LED lighting - Kitchen | 1 | each | Under cabinet LED lighting - Kitchen — 1 each |
| Vanity - Light Fixture - Bath 1 | 2 | each | Vanity - Light Fixture - Bath 1 Allowance. |
| Vanity - Light Fixture - Bath 2 | 1 | each | Vanity - Light Fixture - Bath 2 Allowance. |
| Vanity - Light Fixture - Bath 3 | 1 | each | Vanity - Light Fixture - Bath 3 Allowance. |
| Vanity - Light Fixture - Bath 4 | 1 | each | Vanity - Light Fixture - Bath 4 Allowance. |
| Fans/Lights | 8 | sqft | Fans/Lights Allowance. |
| Can Lights | 45 | sqft | Can Lights Allowance. |
| Can Light- add wiring | 45 | sqft | Can Light- add wiring — 45 ft2 |
| Permanent power connection from transformer | 1 | each | Permanent power service connection. — 1 each |

SPECIAL NOTES

NOTE: Industrial-modern design may call for exposed conduit, which is pricier — confirm with J & J and get a quote (high-scatter line). 400A service and LV allowance are carried.

APPLICABLE CODES & STANDARDS

- NFPA 70 — National Electrical Code (NEC) 2020

- Georgia State Minimum Standard Electrical Code

- 2018 IRC R314 — Smoke alarms; R315 — CO alarms

- Newton County — Electrical permit and inspection

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm service size/location and meter base with utility and J & J.

- Confirm lighting plan, device locations, and any exposed-conduit design.

- Coordinate panel and rough-in with framing and exposed-ceiling design.

COMPLETION — Before calling for inspection or payment

- Service energized; panel labeled; all circuits tested.

- Devices, fixtures, and smoke/CO alarms installed and functional.

- Rough-in and final electrical inspections passed.

- Low-voltage rough-in coordinated and documented.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 12 — Insulation | Open-Cell Spray Foam · Walls & Roof Deck |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish and install spray-foam insulation at walls and roof deck. Roof deck is an unvented low-slope assembly with the foam left exposed (then sprayed black — Scope 17), so foam must be installed to a clean, consistent finished appearance.

SCOPE INCLUDES

- Spray-foam insulation at walls (≈7,986 SF) — allowance.

- Spray-foam insulation at roof deck / ceiling (≈5,585 SF) — allowance.

- Achieve Georgia Climate Zone 3 R-value minimums.

- Mask and protect adjacent surfaces; trim foam clean at exposed areas.

SCOPE EXCLUDES

- Ignition-barrier / black spray-coat over exposed foam (Scope 17).

- Drywall (Scope 13).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Insulation (SqFt) - Walls | 7986 | sqft | Open-cell spray foam insulation, walls. Allowance. |
| Insulation Ceiling/Roof | 5585 | sqft | Open-cell spray foam insulation, roof deck. Allowance. |

SPECIAL NOTES

NOTE: CONFIRM open-cell vs. closed-cell. An unvented low-slope roof deck typically requires closed-cell at the deck; walls may be open-cell. Because foam is exposed, the finished spray must be uniform. Confirm the code-compliant ignition-barrier path with Newton County.

APPLICABLE CODES & STANDARDS

- Georgia Energy Code — R-value minimums (Climate Zone 3)

- 2018 IRC Chapter 11 — Energy Efficiency

- 2018 IRC R316 — Foam plastic / ignition & thermal barrier requirements

- 2018 IRC R302.11 — Fireblocking

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm open-cell vs. closed-cell at walls and roof deck.

- Confirm unvented roof assembly detail and ignition-barrier path with the county.

- Confirm all rough-ins are inspected and complete before foam.

COMPLETION — Before calling for inspection or payment

- Coverage and depth meet specified R-values; no voids.

- Exposed foam trimmed clean and uniform, ready for black coating.

- Insulation inspection passed.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 13 — Drywall | Level 4 · Walls Only · Exposed Ceilings |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish all labor and materials to hang, tape, and finish drywall to Level 4 on interior walls. Ceilings are LEFT EXPOSED per the design (exposed structure, ducts, and foam) — no ceiling board.

SCOPE INCLUDES

- Hang, tape, and finish drywall Level 4 on interior walls (≈17,939 SF).

- Corner bead, fasteners, and joint treatment.

- Garage fire-separation assembly per code where required.

- Sand and prep walls ready for paint (Scope 17).

SCOPE EXCLUDES

- Ceiling drywall — ceilings remain exposed (Scopes 07/10/12 finish).

- Painting (Scope 17); insulation (Scope 12).

- Tile backer/cement board at wet walls (Scope 18).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Drywall - Level 4 | 17939 | sqft | Drywall hung, taped, finished (Level 4) on interior walls; ceilings left exposed per design. — 17,939 sq ft |

SPECIAL NOTES

NOTE: Ceilings are intentionally exposed — do NOT board ceilings. Wall terminations at the exposed ceiling/structure must be clean and detailed since the transition is visible.

APPLICABLE CODES & STANDARDS

- 2018 IRC Section R702 — Interior Covering

- 2018 IRC R302.6 — Dwelling/garage fire separation

- ASTM C840 — Application of gypsum board

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm insulation and all in-wall rough-ins are inspected and complete.

- Confirm wall-to-exposed-ceiling termination detail with J & J.

- Confirm garage fire-separation requirements.

COMPLETION — Before calling for inspection or payment

- Walls finished Level 4, smooth, and ready for paint.

- Garage fire separation complete and inspected.

- Exposed-ceiling terminations clean; no damage to exposed structure.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 14 — Exterior Finishes — Metal Cladding | Standing-Seam Metal Siding · Fascia · Porch Ceilings |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish and install standing-seam metal wall cladding (factory-finished, no field paint), metal/vinyl fascia, and tongue-and-groove covered-porch ceilings. All cladding is standing-seam metal per the modern design.

SCOPE INCLUDES

- Standing-seam metal wall cladding, factory-finished, ≈4,682 SF (incl. waste).

- Weather-resistive barrier and all trims, closures, and flashings.

- Fascia (metal and vinyl), ≈340 LF.

- Tongue-and-groove covered-porch ceilings, ≈382 SF.

SCOPE EXCLUDES

- Metal roofing (Scope 08).

- Exterior paint — none; cladding is factory-finished.

- Windows and exterior doors (Scope 15).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Siding (Vertical) | 4682 | sqft | Standing-seam metal wall cladding, factory-finished (no field paint). — 4,682 ft2 (incl. 10% waste) |
| Fascia (Metal and Vinyl) | 340 | LF | Fascia (Metal and Vinyl) — 340 feet (incl. 10% waste) |
| Porch Ceilings | 381.7 | sqft | Tongue-and-groove ceiling at covered porches. — 381.7 ft2 (incl. 10% waste) |

SPECIAL NOTES

NOTE: Modern metal cladding is chronically under-budgeted — confirm panel/profile and obtain a quote. Coordinate cladding-to-window/door flashing with Scope 15. Confirm whether porch ceilings are T&G wood or exposed metal soffit per the all-metal aesthetic.

APPLICABLE CODES & STANDARDS

- 2018 IRC Chapter 7 — Wall Covering (R703)

- 2018 IRC R703.2 — Water-resistive barrier

- Manufacturer installation instructions for standing-seam wall panels

- Newton County — Housewrap/WRB inspection

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm panel profile, finish color, and trim details with J & J.

- Confirm WRB and window/door flashing sequence before cladding.

- Confirm porch ceiling material (T&G wood vs. metal soffit).

COMPLETION — Before calling for inspection or payment

- Cladding installed plumb, true, and watertight; trims and closures complete.

- Fascia straight and continuous; porch ceilings complete.

- No oil-canning or finish damage; touch-up per manufacturer.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 15 — Windows & Exterior Doors | Black Low-E Windows · Multi-Slide · Entry Doors |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish and install all windows and exterior doors, including a double front entry, hinged exterior doors, and large sliding/multi-slide glass doors. Windows are black-exterior, Low-E, double-pane, no grid (modern). Window/door units are allowances.

SCOPE INCLUDES

- 24 windows — vinyl single-hung & fixed, black exterior, Low-E double-pane, no grid (allowance).

- Window flashing tape and installation labor (24 ea).

- Double front entry door + lockset, modern (allowance) + install.

- 2 single + 2 double exterior doors (black glass) + locksets (allowance) + install.

- 8' sliding glass door and 12' multi-slide door to rear porch (allowance) + install.

SCOPE EXCLUDES

- Interior doors and hardware (Scope 16).

- Garage doors (Scope 24).

- Metal cladding and exterior trim/flashing integration (coordinate with Scope 14).

- Shower/mirror glass (Scope 23).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Windows | 24 | each | Vinyl single-hung & fixed windows, black exterior, Low-E double-pane, no grid (modern). Allowance. |
| Window Tape | 8 | roll | Window Tape — 8 roll |
| Windows - Labor to Install | 24 | each | Windows - Labor to Install — 24 each |
| Front Door - Double (Wood Door) | 1 | each | Double entry door, modern. Allowance. |
| Front Door - Lockset | 1 | each | Front Door - Lockset Allowance. |
| Front Door - Labor to install Door | 1 | each | Front Door - Labor to install Door — 1 each |
| Front Door - Lockset installation | 1 | each | Front Door - Lockset installation — 1 each |
| Exterior Doors - Single Door | 2 | each | Exterior single hinged doors. Allowance. |
| Exterior Doors - Double Door | 2 | each | Exterior double glass doors, black. Allowance. |
| Exterior Doors - 10' Sliding Door | 1 | each | 8' sliding glass door, black. Allowance. |
| Exterior Doors - 12' Sliding Door | 1 | each | 12' sliding/multi-slide glass door to rear porch, black. Allowance. |
| Exterior Door Lockset | 5 | each | Handle and lockset for all hinged entry doors (Excluding Front Door) Allowance. |
| Exterior Doors - Labor to install sliding doors | 2 | each | Exterior Doors - Labor to install sliding doors — 2 each |

SPECIAL NOTES

NOTE: Confirm exact window count and grade (≈14 plan + ≈10 clerestory band) — do not over-price tall/clerestory windows. Multi-slide door may run $8–12k; confirm the door schedule and unit sizes before ordering.

APPLICABLE CODES & STANDARDS

- AAMA 2400 — Window Installation Standard

- Georgia Energy Code — Window U-factor & SHGC, Climate Zone 3

- 2018 IRC R308 — Safety glazing requirements

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm window/door schedule, sizes, swing, and finishes against the plans.

- Confirm rough openings with framer (Scope 07).

- Coordinate flashing sequence with metal cladding (Scope 14).

COMPLETION — Before calling for inspection or payment

- All units installed plumb, level, square; operate smoothly; locked and weather-sealed.

- Flashing integrated with WRB/cladding; no leaks.

- Energy-rating labels retained for inspection; glazing meets safety requirements.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 16 — Interior Trim & Carpentry | Flat-Stock Trim · Interior Doors · Hardware |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish material and labor for interior finish carpentry: modern flat-stock base and window casing, interior doors and hardware, and trim/hardware installation. J & J furnishes door slabs and hardware allowances; contractor provides installation labor and trim materials as noted.

SCOPE INCLUDES

- Modern flat-stock base molding (≈750 LF, allowance) and 1x4 window casing/jambs (≈400 LF).

- Trim labor (≈1,200 LF) and quarter-round / bath / door / cabinet hardware install (≈3,841 SF basis).

- Hang interior doors: 13 hollow-core 8' + 9 solid-core 8' (bedrooms) + 5 6'8" hinged + 8' garage entry door.

- Install door knobs (28), door stops (28), and door hardware.

- Install bath accessories/towel bars and pantry shelving (coordinate).

SCOPE EXCLUDES

- Cabinets (Scope 19) and countertops (Scope 20).

- Exterior doors (Scope 15); garage doors (Scope 24).

- Painting/finishing of trim (Scope 17).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Exterior Doors - Labor to Install Hinged doors | 3 | each | N/A if framing crew installs — 3 each |
| 6'8" Hinged | 5 | each | 6'8" Hinged — 5 each |
| 8'0" Single | 13 | each | Hollow-core flat-stock 8' doors. — 13 each |
| 8'0" Single | 9 | each | Solid-core flat-stock 8' doors at bedrooms. — 9 each |
| 8' Garage Entry Door | 1 | each | 8' Garage Entry Door — 1 each |
| Door knobs | 28 | sqft | Door knobs Allowance. |
| Door Stops | 28 | sqft | Door Stops — 28 ft2 |
| Interior Door Hardware Installation | 28 | sqft | Interior Door Hardware Installation — 28 ft2 |
| Base molding - Choose size | 750 | LF | Modern flat-stock base trim. Allowance. |
| Trim - 1x4 Window casing and Jambs | 400 | LF | Trim - 1x4 Window casing and Jambs — 400 LF (incl. 10% waste) |
| Trim Labor | 1200 | LF | Trim Labor — 1,200 feet (incl. 10% waste) |
| Trim - Labor (Install Quarter Round & Bath, Door, Cabinet Hardware) | 3841 | sqft | Trim - Labor (Install Quarter Round & Bath, Door, Cabinet Hardware) — 3,841 sq ft (incl. 10% waste) |

SPECIAL NOTES

NOTE: Modern flat-stock profile — confirm base size and casing detail. Bedrooms get solid-core 8' doors; balance hollow-core flat-stock. Confirm door and hardware counts against the plan.

APPLICABLE CODES & STANDARDS

- 2018 IRC R311.7 — Stairways (if applicable)

- 2018 IRC R312 — Guards

- 2018 IRC R302.5 — Dwelling/garage door opening protection

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm trim profiles, door styles/cores, and hardware finishes with J & J.

- Confirm door and casing counts against the plan.

- Confirm walls are painted/primed status for install sequence.

COMPLETION — Before calling for inspection or payment

- All doors hung plumb; operate and latch correctly; hardware set.

- Trim tight, mitered clean, and caulk-ready; no gaps.

- Accessories and shelving installed level and secure.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 17 — Painting & Exposed-Structure Coating | Interior Walls · Black Spray-Coat Structure/Duct/Foam |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish all labor and materials to paint interior walls and to spray-coat the exposed ceiling structure, ductwork, and spray-foam insulation BLACK as an industrial finish — including the code-required ignition-barrier coating over exposed foam.

SCOPE INCLUDES

- Paint interior walls (≈17,939 SF) — prime + finish coats.

- Spray-coat exposed structure / ducts / foam black (≈3,841 SF), including ignition-barrier coating over exposed foam.

- Mask and protect floors (stained slab), windows, cabinets, and fixtures.

- Caulk and prep trim; final touch-up.

SCOPE EXCLUDES

- Exterior finishes — metal cladding is factory-finished (Scope 14).

- Concrete floor staining/sealing (Scope 21).

- Cabinet and countertop finishing (Scopes 19 / 20).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Paint/Stain - Interior SF | 17939 | sqft | Interior walls painted. — 17,939 sq ft |
| Spray-coat exposed structure / ducts / foam - BLACK | 3841 | sqft | Exposed ceiling structure, ductwork and insulation spray-coated black (industrial finish); includes code ignition-barrier coating. — 3,841 ft2 (incl. 10% waste) |

SPECIAL NOTES

NOTE: The black spray-coat over exposed foam must serve as / be compatible with a code-listed ignition barrier — CONFIRM the product and code path with Newton County before spraying. Protect the stained-concrete floor and exposed ductwork finish during all painting.

APPLICABLE CODES & STANDARDS

- 2018 IRC R316 — Foam plastic ignition/thermal barrier (intumescent coating per listing)

- 2018 IRC R702 — Interior wall finish

- Manufacturer application specifications; low-VOC per Georgia requirements

- Coating manufacturer ESR/listing for ignition-barrier use over spray foam

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm wall colors/sheens and the black coating product (ignition-barrier listing).

- Confirm drywall is finished and exposed structure/duct is complete and clean.

- Protect stained-concrete floors and exposed metal duct before spraying.

COMPLETION — Before calling for inspection or payment

- Walls uniformly coated, no holidays/lap marks.

- Exposed structure/duct/foam fully and evenly coated black; ignition-barrier coverage rate documented.

- No overspray on floors, glass, cabinets, or fixtures.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 18 — Tile | Kitchen Backsplash · 4 Tiled Baths · Showers |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish labor and material allowances to install all tile: kitchen backsplash and fully-tiled bath walls, showers (mud beds, benches, niches, Schluter), and bath floors across all 4 baths. Tile material is owner allowance; labor and sundries by contractor.

SCOPE INCLUDES

- Kitchen backsplash — tile + sundries + labor (≈45–50 SF).

- Bath 1 (master): tile walls + Schluter + shower mud bed, bench, niche + floor tile.

- Baths 2–4: tile walls, shower mud beds, shower floor tile, floor tile.

- All thinset, grout, spacers, sealers, and waterproofing membrane.

- Waterproof and slope all shower pans per code.

SCOPE EXCLUDES

- Stained-concrete main floors (Scope 21).

- Plumbing fixtures, valves, and drains (Scope 09).

- Shower glass enclosures and mirrors (Scope 23).

- Vanity cabinets and tops (Scopes 19 / 20).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Backsplash - Material allowance | 49.5 | sqft | Allowance. |
| Tile sundries - Glue Grout, Spacers, etc | 49.5 | sqft | Tile sundries - Glue Grout, Spacers, etc — 49.5 each |
| Backsplash - Tile labor | 45 | sqft | Backsplash - Tile labor — 45 each |
| Tile Walls - Material allowance - Bath 1 | 143 | sqft | Allowance. |
| Tile sundries - Glue Grout, Spacers, etc - Bath 1 | 143 | sqft | Tile sundries - Glue Grout, Spacers, etc - Bath 1 — 143 ft2 (incl. 10% waste) |
| Schluter - Bath 1 | 143 | sqft | Schluter - Bath 1 — 143 sq ft (incl. 10% waste) |
| Tile wall labor - Bath 1 | 130 | sqft | Tile wall labor - Bath 1 — 130 ft2 (incl. 10% waste) |
| Niche Labor - Bath 1 | 1 | each | Niche Labor - Bath 1 — 1 each |
| Niche tile allowance - Bath 1 | 3 | LF | Allowance. |
| Shower bench - Bath 1 | 1 | each | Shower bench - Bath 1 — 1 each |
| Shower Bench Labor - Bath 1 | 1 | each | Shower Bench Labor - Bath 1 — 1 each |
| Shower mud bed - Bath 1 | 40 | sqft | Shower mud bed - Bath 1 — 40 sq ft (incl. 10% waste) |
| Shower floor tile allowance - Bath 1 | 40 | sqft | Allowance. |
| Flooring Tile - Bath 1 | 77 | sqft | Flooring Tile - Bath 1 Allowance. |
| Flooring tile sundries - Bath 1 | 77 | sqft | Flooring tile sundries - Bath 1 — 77 each |
| Flooring tile Labor - Bath 1 | 70 | sqft | Flooring tile Labor - Bath 1 — 70 each |
| Tile Walls - Material allowance - Bath 2 | 90 | sqft | Allowance. |
| Tile wall labor - Bath 2 | 90 | sqft | Tile wall labor - Bath 2 — 90 ft2 (incl. 10% waste) |
| Shower mud bed - Bath 2 | 36 | sqft | Shower mud bed - Bath 2 — 36 sq ft (incl. 10% waste) |
| Shower floor tile allowance - Bath 2 | 36 | sqft | Allowance. |
| Flooring Tile - Bath 2 | 45 | sqft | Flooring Tile - Bath 2 Allowance. |
| Tile Walls - Material allowance - Bath 3 | 90 | sqft | Allowance. |
| Tile wall labor - Bath 3 | 90 | sqft | Tile wall labor - Bath 3 — 90 ft2 (incl. 10% waste) |
| Shower mud bed - Bath 3 | 36 | sqft | Shower mud bed - Bath 3 — 36 sq ft (incl. 10% waste) |
| Shower floor tile allowance - Bath 3 | 36 | sqft | Allowance. |
| Flooring Tile - Bath 3 | 45 | sqft | Flooring Tile - Bath 3 Allowance. |
| Flooring tile sundries - Bath 3 | 45 | sqft | Flooring tile sundries - Bath 3 — 45 each |
| Flooring tile Labor - Bath 3 | 45 | sqft | Flooring tile Labor - Bath 3 — 45 each |
| Tile Walls - Material allowance - Bath 4 | 90 | sqft | Allowance. |
| Tile wall labor - Bath 4 | 90 | sqft | Tile wall labor - Bath 4 — 90 ft2 (incl. 10% waste) |
| Shower mud bed - Bath 4 | 36 | sqft | Shower mud bed - Bath 4 — 36 sq ft (incl. 10% waste) |
| Shower floor tile allowance - Bath 4 | 36 | sqft | Allowance. |
| Flooring Tile - Bath 4 | 45 | sqft | Flooring Tile - Bath 4 Allowance. |
| Flooring tile sundries - Bath 4 | 45 | sqft | Flooring tile sundries - Bath 4 — 45 each |
| Flooring tile Labor - Bath 4 | 45 | sqft | Flooring tile Labor - Bath 4 — 45 each |

SPECIAL NOTES

NOTE: Finish tier is HIGH-END (fully-tiled showers, benches, niches). Confirm tile selections, layout, and grout colors before starting. Coordinate shower pan waterproofing with plumbing valve/drain rough-in.

APPLICABLE CODES & STANDARDS

- ANSI A108 — Ceramic Tile Installation

- TCNA Handbook for Ceramic, Glass, and Stone Tile Installation

- ANSI A118.10 — Waterproof membranes

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm tile selections, layouts, niches, and bench locations with J & J/owner.

- Confirm shower valve/drain rough-in is set and inspected (Scope 09).

- Confirm wall substrate (cement board/membrane) at wet areas.

COMPLETION — Before calling for inspection or payment

- All showers flood-tested watertight before tile where applicable.

- Tile flat, aligned, full-coverage thinset; grout uniform and sealed.

- Niches, benches, and Schluter edges clean and square.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 19 — Cabinetry | Kitchen · 4 Vanities · Supply & Install |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish and install high-end cabinetry: kitchen lowers/uppers/island/tall units/vent-hood cabinet and vanity cabinets for all 4 baths, including hardware and installation labor. Cabinet and vanity material are owner allowances.

SCOPE INCLUDES

- Kitchen: 20 LF lowers, 12 LF uppers, 10 LF island, 6 LF tall (fridge/oven/linen), vent-hood cabinet (allowance).

- Laundry/utility: lowers (12 LF) + uppers (10 LF) + counter.

- Vanity cabinets baths 1–4 + cabinet hardware (allowance).

- Labor to assemble and install all cabinets and vanities; install towel bars.

SCOPE EXCLUDES

- Countertops (Scope 20).

- Plumbing fixtures/faucets (Scope 09); mirrors (Scope 23).

- Appliances and appliance install (by others / owner-supplied).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| LF of Lowers - Kitchen | 20 | LF | High-end cabinetry, kitchen. Allowance. |
| LF of Uppers - Kitchen | 12 | LF | LF of Uppers - Kitchen Allowance. |
| LF of Island - Kitchen | 10 | LF | LF of Island - Kitchen Allowance. |
| LF of Tall Cabinet (Fridge, Oven, Linen) - Kitchen | 6 | LF | LF of Tall Cabinet (Fridge, Oven, Linen) - Kitchen Allowance. |
| Each Vent Hood Cabinet - Kitchen | 1 | each | Each Vent Hood Cabinet - Kitchen Allowance. |
| Labor to assemble and install cabinets - Kitchen | 40 | each | Labor to assemble and install cabinets - Kitchen — 40 each |
| LF of Lowers | 12 | LF | LF of Lowers Allowance. |
| LF of Uppers | 10 | LF | LF of Uppers Allowance. |
| Countertops | 15 | sqft | Countertops — 15 ft2 (incl. 1.24% waste) |
| Vanity Cabinets - Bath 1 | 18 | sqft | Vanity Cabinets - Bath 1 Allowance. |
| Vanity Cabinets - Hardware - Bath 1 | 18 | sqft | Vanity Cabinets - Hardware - Bath 1 Allowance. |
| Vanity Cabinets - Installation - Bath 1 | 18 | sqft | Vanity Cabinets - Installation - Bath 1 — 18 ft2 |
| Install towel bars - Bath 1 | 2 | each | Install towel bars - Bath 1 — 2 each |
| Vanity Cabinets - Bath 2 | 9 | sqft | Vanity Cabinets - Bath 2 Allowance. |
| Vanity Cabinets - Hardware - Bath 2 | 9 | sqft | Vanity Cabinets - Hardware - Bath 2 Allowance. |
| Vanity Cabinets - Installation - Bath 2 | 9 | sqft | Vanity Cabinets - Installation - Bath 2 — 9 ft2 |
| Install towel bars - Bath 2 | 1 | each | Install towel bars - Bath 2 — 1 each |
| Vanity Cabinets - Bath 3 | 9 | sqft | Vanity Cabinets - Bath 3 Allowance. |
| Vanity Cabinets - Hardware - Bath 3 | 9 | sqft | Vanity Cabinets - Hardware - Bath 3 Allowance. |
| Vanity Cabinets - Installation - Bath 3 | 9 | sqft | Vanity Cabinets - Installation - Bath 3 — 9 ft2 |
| Install towel bars - Bath 3 | 1 | each | Install towel bars - Bath 3 — 1 each |
| Vanity Cabinets - Bath 4 | 9 | sqft | Vanity Cabinets - Bath 4 Allowance. |
| Vanity Cabinets - Hardware - Bath 4 | 9 | sqft | Vanity Cabinets - Hardware - Bath 4 Allowance. |
| Vanity Cabinets - Installation - Bath 4 | 9 | sqft | Vanity Cabinets - Installation - Bath 4 — 9 ft2 |
| Install towel bars - Bath 4 | 1 | each | Install towel bars - Bath 4 — 1 each |

SPECIAL NOTES

NOTE: High-end cabinetry allowance ($200/LF kitchen). Confirm door style, finish, and box construction. Coordinate vent-hood cabinet with kitchen appliance/hood selection and HVAC.

APPLICABLE CODES & STANDARDS

- ANSI/KCMA A161.1 — Cabinet standards

- CARB ATCM 93120 — Formaldehyde emissions compliance

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm cabinet layout, sizes, door style, and finish against the plan/selections.

- Confirm appliance and fixture dimensions before setting cabinets.

- Confirm walls/floors are complete (stained slab protected).

COMPLETION — Before calling for inspection or payment

- Cabinets level, plumb, securely fastened; doors/drawers aligned.

- Hardware installed; fillers and scribes clean.

- Ready to template for countertops (Scope 20).

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 20 — Countertops | Level-6 Quartz / Granite · Kitchen & Baths |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to template, fabricate, and install Level-6 granite/quartz (or concrete) countertops at the kitchen and all 4 bath vanities, including cutouts and edge profiles. Countertop material is owner allowance.

SCOPE INCLUDES

- Kitchen countertops — Level-6 granite/quartz, ≈80 SF (allowance) + cutouts.

- Bath 1–4 vanity tops — Level-6 quartz, ≈14–32 SF each (allowance) + cutouts.

- Template, fabricate, deliver, and install; seam and seal.

- Sink/faucet cutouts coordinated with plumbing (Scope 09).

SCOPE EXCLUDES

- Cabinets and vanities (Scope 19).

- Sinks, faucets, and plumbing connections (Scope 09).

- Backsplash tile (Scope 18).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Granite or Quartz - Kitchen | 80 | sqft | Level-6 granite/quartz (or concrete) countertops, fabricated & installed. Allowance. |
| Charge per cutout/hole | 1 | each | Charge per cutout/hole — 1 each |
| Charge per cutout/hole - Bath 1 | 2 | each | Charge per cutout/hole - Bath 1 — 2 each |
| Granite or Quartz - Bath 1 | 32 | sqft | Level-6 quartz vanity tops. Allowance. |
| Charge per cutout/hole - Bath 2 | 1 | each | Comes with vanity — 1 each |
| Granite or Quartz - Bath 2 | 14 | sqft | Comes with vanity Allowance. |
| Charge per cutout/hole - Bath 3 | 1 | each | Charge per cutout/hole - Bath 3 — 1 each |
| Granite or Quartz - Bath 3 | 14 | sqft | Granite or Quartz - Bath 3 Allowance. |
| Charge per cutout/hole - Bath 4 | 1 | each | Charge per cutout/hole - Bath 4 — 1 each |
| Granite or Quartz - Bath 4 | 14 | sqft | Granite or Quartz - Bath 4 Allowance. |

SPECIAL NOTES

NOTE: High-end finish tier — confirm slab material, color, edge profile, and seam locations before fabrication. Template only after cabinets are set and verified level.

APPLICABLE CODES & STANDARDS

- MIA Dimension Stone Design Manual

- ANSI/NSF 51 — Food equipment materials

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm material, color, edge profile, and overhangs with J & J/owner.

- Confirm cabinets are set and level before templating.

- Confirm sink/faucet/cooktop cutout specs with plumbing/appliances.

COMPLETION — Before calling for inspection or payment

- Tops installed level, fully supported; seams tight and inconspicuous.

- Cutouts accurate; edges polished per spec; sealed if natural stone.

- No chips or scratches; cleaned and protected.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 21 — Flooring — Stained & Sealed Concrete | Polished/Stained Slab · Finished Floor Throughout |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to grind, stain, and seal the concrete slab as the finished floor throughout the living areas (≈3,631 SF). The slab IS the finish floor per the design — no LVP or wood. High-end grind/stain/seal.

SCOPE INCLUDES

- Surface prep/grind of slab to receive stain.

- Stain and seal concrete floors, ≈3,631 SF.

- Multiple seal/wear coats per specification.

- Mask and protect walls, cabinets, and adjacent finishes.

SCOPE EXCLUDES

- Slab placement and finishing (Scope 05).

- Bath floor tile (Scope 18).

- Wall painting (Scope 17).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Stained & sealed concrete floors | 3631 | sqft | Stained and sealed concrete finished floors throughout (polished/sealed slab). — 3,631 ft2 |

SPECIAL NOTES

NOTE: Confirm finish (stain color vs. polish) and sheen with owner. Slab finish from Scope 05 must be clean and crack-controlled — coordinate timing so the finished floor is protected through the balance of construction.

APPLICABLE CODES & STANDARDS

- ASTM F2170 — RH testing in concrete slabs

- ASTM F1869 — Moisture vapor emission rate

- Stain/sealer manufacturer installation specifications

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm stain color, sheen, and sealer system with J & J/owner.

- Confirm slab moisture is within sealer tolerances (test).

- Confirm slab is clean and free of defects before grinding.

COMPLETION — Before calling for inspection or payment

- Floors uniformly stained and sealed; consistent color/sheen.

- No roller/lap marks, bubbles, or contamination.

- Floor protected from subsequent trades until handover.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 22 — Masonry — Stone Veneer | Adhered Stone Accent Wall |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish and install adhered stone veneer at the accent wall (≈282 SF), including substrate prep, lath, scratch coat, setting bed, and grouting/pointing as the selected stone requires.

SCOPE INCLUDES

- Furnish stone veneer material, ≈282 SF (allowance).

- Install lath, weather-resistive barrier, scratch coat as required.

- Set stone veneer; point/grout joints per stone type.

- Clean and seal as specified.

SCOPE EXCLUDES

- Fireplace and chimney (none on this project).

- Metal cladding and other exterior finishes (Scope 14).

- Framing/substrate (Scope 07).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Stone Material | 282 | sqft | Stone veneer accent wall. Allowance. |
| Stone Labor | 282 | sqft | Stone Labor — 282 sq ft |

SPECIAL NOTES

NOTE: Confirm exact location (entry accent wall) and stone selection. Confirm whether veneer is interior or exterior to set the correct WRB/flashing detail. Carried as a stone-veneer accent only — no fireplace.

APPLICABLE CODES & STANDARDS

- 2018 IRC R703.12 — Adhered masonry veneer

- TMS 402/602 — Building Code Requirements & Specification for Masonry Structures

- ASTM C1670 — Adhered manufactured stone masonry veneer units

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm stone selection, location, and extent with J & J/owner.

- Confirm substrate and WRB/flashing detail (interior vs. exterior).

- Confirm framing/blocking is adequate for veneer weight.

COMPLETION — Before calling for inspection or payment

- Veneer fully bonded; no loose units; joints pointed clean.

- WRB/flashing intact behind veneer (if exterior).

- Sealed and cleaned per spec; no staining of adjacent finishes.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 23 — Glass & Shower Enclosures | Frameless Shower Glass · Vanity Mirrors |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish and install frameless glass shower enclosures and vanity mirrors at all baths. Master and additional showers receive frameless tempered-glass enclosures; each vanity receives a mirror.

SCOPE INCLUDES

- 2 frameless glass shower enclosures (allowance), tempered safety glass.

- Vanity mirrors: 2 at bath 1, 1 each at baths 2–4 (allowance) + installation.

- All clips, hinges, channels, and sealant.

- Measure after tile is complete (Scope 18).

SCOPE EXCLUDES

- Tile and shower pans (Scope 18).

- Plumbing fixtures and valves (Scope 09).

- Windows and exterior glass doors (Scope 15).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Vanity Sink - Mirror - Bath 1 | 2 | each | Vanity Sink - Mirror - Bath 1 Allowance. |
| Install Mirror - Bath 1 | 2 | each | Install Mirror - Bath 1 — 2 each |
| Vanity Sink - Mirror - Bath 2 | 1 | each | Vanity Sink - Mirror - Bath 2 Allowance. |
| Install Mirror - Bath 2 | 1 | each | Install Mirror - Bath 2 — 1 each |
| Vanity Sink - Mirror - Bath 3 | 1 | each | Vanity Sink - Mirror - Bath 3 Allowance. |
| Install Mirror - Bath 3 | 1 | each | Install Mirror - Bath 3 — 1 each |
| Vanity Sink - Mirror - Bath 4 | 1 | each | Vanity Sink - Mirror - Bath 4 Allowance. |
| Install Mirror - Bath 4 | 1 | each | Install Mirror - Bath 4 — 1 each |
| Glass Shower Enclosure - Frameless (Master) | 2 | each | Frameless glass shower enclosure(s). Allowance. |

SPECIAL NOTES

NOTE: Field-measure frameless enclosures only AFTER tile is complete and verified plumb. All glass must be tempered safety glass per code.

APPLICABLE CODES & STANDARDS

- 2018 IRC R308 — Safety glazing (hazardous locations)

- ANSI Z97.1 — Safety glazing materials

- CPSC 16 CFR 1201 — Safety standard for architectural glazing

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm enclosure configurations and hardware finishes with J & J/owner.

- Confirm tile walls/curbs are complete, plumb, and waterproof before measuring.

- Confirm mirror sizes and locations against vanities.

COMPLETION — Before calling for inspection or payment

- Enclosures installed plumb; doors swing/seal correctly; no leaks.

- All glass tempered and labeled; clips/sealant clean.

- Mirrors level and securely mounted.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 24 — Garage Doors | Modern Flush 16'x8' · Opener |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish and install a modern flush 16'x8' double garage door with opener at the conditioned garage, including tracks, springs, weatherstripping, and operator.

SCOPE INCLUDES

- Modern flush 16'x8' double garage door (allowance).

- Garage door opener (allowance) + installation.

- Tracks, springs, hardware, and weatherstripping.

- Test and adjust operation and safety reverse.

SCOPE EXCLUDES

- Garage entry (man) door (Scope 16).

- Electrical outlet/wiring for opener (Scope 11).

- Garage HVAC (Scope 10).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Double Garage Door | 1 | sqft | Modern flush 16'x8' garage door. Allowance. |
| Double Garage Door Opener | 1 | sqft | Double Garage Door Opener Allowance. |
| Double Garage Door Installation | 1 | sqft | Double Garage Door Installation — 1 ft2 |

SPECIAL NOTES

NOTE: Garage is conditioned — confirm insulated door and weatherstripping for the conditioned space. Confirm flush modern panel style and color.

APPLICABLE CODES & STANDARDS

- UL 325 — Door operators (entrapment protection)

- ANSI/DASMA 102 — Sectional Doors

- 2018 IRC R309 — Garages

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm door style, color, insulation value, and opener with J & J/owner.

- Confirm rough opening and headroom with framing.

- Confirm opener power/outlet is available (Scope 11).

COMPLETION — Before calling for inspection or payment

- Door operates smoothly; balanced springs; weatherseal complete.

- Opener safety reverse and photo-eyes functional.

- No damage to finish; remotes/keypads provided.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 26B — Driveway & Concrete Walkways | End-of-Job · 4" Concrete Drive |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish all labor, equipment, and materials for the final concrete driveway and walkways (≈2,400 SF, 4" slab). End-of-job scope performed after exterior work is complete to avoid construction damage.

SCOPE INCLUDES

- Fine-grade and prep drive/walk subgrade.

- Form, place, and finish 4" concrete driveway and walkways (≈2,400 SF; ≈46 CY).

- Labor & equipment for placement and finishing.

- Control joints, broom/finish per spec, and cure.

SCOPE EXCLUDES

- Construction gravel drive and culvert (Scope 01).

- Final grading, sod, and landscaping (Scope 27).

- Building slab (Scope 05).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Labor & Equipment | 1 | each | Labor & Equipment — 1 each |
| Labor | 2400 | sqft | Labor Allowance. |
| Concrete | 46.2 | CY | 4" Slab Allowance. |

SPECIAL NOTES

NOTE: Drive length (~110 LF) and area were estimated and must be FIELD-CONFIRMED before pour. GDOT/county driveway entrance requirements apply at the county road. End-of-job scope — schedule after exterior trades are complete.

APPLICABLE CODES & STANDARDS

- ACI 302.1R — Guide for Concrete Floor and Slab Construction

- 2018 IRC R301.1 — Application of design criteria

- GDOT driveway entrance requirements at county road

- Newton County — Driveway permit requirements (if applicable)

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Field-verify drive layout, length, width, and grade with J & J.

- Confirm entrance/culvert and county driveway permit requirements.

- Confirm exterior work is complete to avoid damaging finished drive.

COMPLETION — Before calling for inspection or payment

- Drive/walks placed to correct grade; positive drainage away from structure.

- Control joints cut; finish uniform; no cracking/spalling.

- Cured and protected; entrance meets county/GDOT requirements.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 27 — Landscaping | Final Grade · Sod · Irrigation · Plants |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to furnish all labor, equipment, and materials for final grading, backfill, lawn establishment (sod/seed), irrigation, and a landscape plant package, achieving final site stabilization.

SCOPE INCLUDES

- Final grading around structure (positive drainage away from foundation).

- Backfill dirt as needed (≈3 loads, allowance).

- Sod / lawn establishment (allowance) and seed & straw on balance.

- Irrigation system (allowance) with backflow preventer.

- Landscape plant package (allowance).

SCOPE EXCLUDES

- Concrete driveway and walkways (Scope 26B).

- Erosion control during construction (Scope 01).

- Septic field final grade (Scope 02).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Final Grading | 1 | sqft | Final Grading — 1 ft2 |
| Backfill Dirt (14Yd Net) | 3 | loads | Backfill Dirt (14Yd Net) Allowance. |
| Sod | 1 | each | Sod / lawn establishment - allowance. |
| Irrigation | 1 | each | Irrigation system - allowance. |
| Plant package | 1 | sqft | Landscape plant package - allowance. |
| Seed and straw | 1 | sqft | Seed and straw Allowance. |

SPECIAL NOTES

NOTE: Confirm scope, plant package, and irrigation extent with owner. Achieve final site stabilization per Georgia EPD before NOT (Notice of Termination).

APPLICABLE CODES & STANDARDS

- Georgia EPD — Final site stabilization requirements

- Newton County — Irrigation backflow preventer inspection

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm final grade plan and drainage with J & J.

- Confirm plant package, sod type, and irrigation zones with owner.

- Confirm hardscape (Scope 26B) is complete.

COMPLETION — Before calling for inspection or payment

- Final grade drains away from foundation; no ponding.

- Sod/seed established; irrigation operational; backflow inspected.

- Site stabilized per EPD; ready for NOT.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 28A — Final Clean — Interior | End-of-Job · Whole-House Detail Clean |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to perform a complete interior final clean of the residence (≈2,637 SF heated) prior to the owner walkthrough — all interior surfaces detailed and move-in ready. All trades must be complete and off-site before cleaning begins.

SCOPE INCLUDES

- Clean all floors (stained concrete + tile), baseboards, and trim.

- Clean countertops, cabinets (inside and out), and all fixtures.

- Clean all bathroom surfaces, mirrors, and glass enclosures.

- Clean light fixtures, switch plates, and interior of windows.

- Remove all construction dust, debris, stickers, and protective films.

SCOPE EXCLUDES

- Exterior surfaces and pressure washing (Scope 28C).

- Exterior window glass (Scope 28B).

- Touch-up painting (Scope 17) and punch repairs (builder).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Cleaning - Final Clean Interior | 3841 | sqft | Cleaning - Final Clean Interior — 3,841 sq ft |

SPECIAL NOTES

NOTE: Use non-damaging agents on the stained-concrete floor and exposed-metal duct/structure. No construction activity after final clean without J & J approval.

APPLICABLE CODES & STANDARDS

- OSHA 29 CFR 1926.57 — Ventilation during cleaning operations

- EPA 40 CFR Part 745 — Lead RRP awareness (new-construction reference)

- Contractor must use non-damaging cleaning agents on all finished surfaces

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm all trades are complete and off-site.

- Confirm finished floor and exposed structure cleaning agents are non-damaging.

- Confirm walkthrough date with J & J.

COMPLETION — Before calling for inspection or payment

- All interior surfaces clean, streak-free, and move-in ready.

- No dust on exposed structure/duct, ledges, or fixtures.

- Passes J & J pre-walkthrough inspection.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 28B — Window Cleaning | End-of-Job · Interior & Exterior Glass |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to clean all window and glass-door glass, interior and exterior (24 windows plus exterior glass doors), removing all construction film, labels, and overspray, and to clean and reinstall screens.

SCOPE INCLUDES

- Clean interior and exterior glass of all 24 windows.

- Clean exterior glass doors and sliding/multi-slide glass.

- Remove stickers, labels, paint overspray, and construction film.

- Clean and reinstall all screens.

SCOPE EXCLUDES

- Interior surface cleaning (Scope 28A).

- Exterior pressure washing (Scope 28C).

- Glazing repair/replacement (builder/Scope 15).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Cleaning - Final window cleaning | 24 | each | Cleaning - Final window cleaning — 24 each |

SPECIAL NOTES

NOTE: All glass is Low-E safety glass — do NOT use abrasive tools or scrapers that scratch coatings. Use non-streaking products and follow ladder/lift safety.

APPLICABLE CODES & STANDARDS

- Contractor must use non-abrasive, non-streaking window cleaning products

- All glass is safety glass per 2018 IRC R308 — do not use abrasive tools

- OSHA 29 CFR 1926 Subpart R — ladder and lift access safety

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm all trades complete; no further overspray expected.

- Confirm safe ladder/lift access for clerestory/high glass.

- Confirm Low-E coating cleaning method.

COMPLETION — Before calling for inspection or payment

- All glass clean and streak-free, interior and exterior.

- No scratches to Low-E coatings; labels/film removed.

- Screens cleaned and reinstalled.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com

| 28C — Exterior Pressure Washing | End-of-Job · Cladding · Porches · Flatwork |
| --- | --- |

| PROJECT Guarino Residence | ADDRESS Lot 11 River Cove Meadows, Monticello, GA | PROJECT ID PR-111 | BUILDER J & J Custom Homes, LLC |
| --- | --- | --- | --- |

SCOPE OVERVIEW

Contractor to pressure wash all exterior surfaces — metal cladding, stone veneer, porches, walks, and driveway — using appropriate pressure for each material, prior to owner walkthrough.

SCOPE INCLUDES

- Pressure wash metal cladding and fascia (lower pressure for finish).

- Pressure wash stone veneer, covered porches, walks, and driveway.

- Remove construction dirt, splatter, and residue.

- Protect electrical fixtures, outlets, and vents from water intrusion.

SCOPE EXCLUDES

- Interior cleaning (Scope 28A) and window glass (Scope 28B).

- Touch-up to cladding finish (Scope 14).

QUANTITIES & SPECIFICATIONS

| Item Description | Quantity | Unit | Notes / Spec |
| --- | --- | --- | --- |
| Cleaning - Exterior Pressure washing | 3841 | each | Cleaning - Exterior Pressure washing — 3,841 each |

SPECIAL NOTES

NOTE: Use the correct PSI per surface — factory-finished metal cladding requires lower pressure than concrete. Protect all electrical fixtures, outlets, and vents; contain wash water away from drainage features per EPD.

APPLICABLE CODES & STANDARDS

- OSHA 29 CFR 1910.137 — Electrical safety during pressure washing near fixtures

- Manufacturer guidelines for pressure ratings on siding, masonry, and trim

- Georgia EPD — Stormwater runoff; contain and direct wash water away from drainage

INSPECTION & QUALITY CHECKLIST

PRE-WORK — Verify before starting

- Confirm correct PSI per surface (cladding vs. concrete).

- Protect electrical fixtures, outlets, and vents.

- Confirm all exterior trades are complete.

COMPLETION — Before calling for inspection or payment

- All exterior surfaces clean; no finish damage or oil-canning.

- No water intrusion at fixtures/vents.

- Site presentable for owner walkthrough.

J & J STANDARD REQUIREMENTS — ALL TRADES

- All work must comply with applicable 2018 IRC, state amendments, and Newton County codes.

- Subcontractor is responsible for all trade-specific permits and inspections (unless noted otherwise).

- Work areas to be cleaned and debris removed at end of each work day.

- Coordinate scheduling with J & J project manager a minimum of 72 hours in advance.

- Any work outside this scope requires a written change order signed by J & J before proceeding.

- Current General Liability and Workers Compensation COI required before work begins.

- All subcontractors must hold a current Georgia license for their trade.

- Verify all field dimensions before ordering or installing. Do not scale from drawings.

- Payment tied to draw schedule. Invoices at draw milestones only.

- Lien waiver (conditional) required with each invoice; final (unconditional) at completion.

Questions: Keli Rector — 404-934-8214 — keli@jnjcustomhomes.com — www.jnjcustomhomes.com